Supplemental GPS Observations

Contents

GPS units	2

Receivers	2

Antennas	2

Analysis software, mode of operation	2

GPS Overview	2

Figure GPS-1:   Observation Sketch	3

Table GPS-1:   GPS session summary	4

GPS Observations	4

Analysis Software	4

Results	5

Table GPS-2:  Comparison of redundant vectors.	6

Attachment:  GPS Geocentric Coordinates and Covariances (SINEX)	7

# GPS units

## Receivers

Four (4) Trimble NetR5

P/N: 62800-00

S/Ns: 4624K01583, 584, 590, and 631

Mfr. Specifications for Static GPS Surveying

Horizontal:	+/- 5 mm + 0.5 ppm RMS

Vertical:	+/- 5 mm + 1 ppm RMS

## Antennas

Four (4) Topcon CR-G3 choke ring antennas		Antenna Code:  TPSCR.G3

S/N: 383-1613, -1614, -1626 and -1628			P/N:  1-044301-01

## Analysis software, mode of operation

Post-processing and adjustment were undertaken using NGS’s beta version of Online Positioning User Service (OPUS) Projects, an interactive web page.  Data is tagged with a project identifier during typical OPUS upload.  OPUS Projects assigns the data to survey points and sorts it into sessions.  Data manager must log in, select a session and reconfigure it if desired before starting the processing.  Processing reports are generated and interactive tables display a summary of results.  Processed base lines are made available for the adjustment phase.  The user is allowed limited flexibility in setting adjustment parameters.  Processing and adjustment results are automatically forwarded to user via email.

# GPS Overview

GPS data were collected on six main-scheme points and retrieved from five IGS tracking stations.   The geometry of the point layout and local obstructions at some of the points caused for a less than ideal observing scheme (see Fig. GPS-1).  Many hours of observations were conducted to help counter these deficiencies.

### Figure GPS-1:   Observation Sketch

NLIB        USNO                            BRMU

RCM2

TABV

RCM4

FLR1

FLR2

FLF1

TF2A

FLR3

INEG

SCUB

Table GPS-1 shows sessions and hours of simultaneous data collection for each session.

| Day of Year (sess. #) | 38 | 39 | 40a | 40b | 42 | 43 | 45 | 46 | 47a | 47b | 48 | 49 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FLF1 | 24 | 24 | 13 | 15 | 24 | 17 | 24 | 24 | 14 | 29 | 17 | 23 |
| FLR1 | 24 | 24 | 13 | 15 |  |  |  |  |  |  | 15 | 22 |
| FLR2 | 24 | 24 | 12 | 15 | 24 | 15 | 24 | 24 | 14 | 29 | 15 | 22 |
| FLR3 | 24 | 24 | 12 | 15 |  |  |  |  |  |  | 15 | 23 |
| TF2A | 24 | 24 | 12 |  |  |  |  |  |  |  |  |  |
| RCM2 |  |  |  |  | 23 | 17 | 24 | 24 | 5 |  | 17 | 22 |
| RCM4 |  |  |  |  | 23 | 16 | 24 | 24 | 14 | 29 |  |  |
| TABV |  |  |  |  | 23 | 17 | 24 | 24 | 14 | 27 |  |  |

### Table GPS-1:   GPS session summary

# GPS Observations

## Analysis Software

NGS’s Online Positioning User’s Service (OPUS) Projects was used to post-process and analyze GPS data and to compute least-squares, 3-D estimates of point positions.

OPUS Projects uses NGS’s Program for Adjustment of GPS Ephemerides (PAGES) for post-processing.  Default settings were used with the following exceptions; 1) hubs were limited to only FLF1; 2) the cutoff elevation for processing was lowered to 10o; 3) ITRF2008 was selected as the output reference frame; 4) USGG2012 was selected as the output geoid model; 5) Piecewise Linear was selected as the tropospheric model with an interval of 7200; and 6) constraint weights were set to Normal.   These and the remaining default settings resulted in:

- the use of IGS08.atx absolute antenna calibration file.

- the use of IGS orbits (+/- 2 cm).

- the use of IGS08solution coordinate reference system with coordinates updated to date of observations using velocities.

- the use of default (modeled) meteorological values.

- the use of dual-frequency ionospheric correction.

- time parameters solved in data reduction.

- a double-difference, ionosphere-free (IF) combination solution, where

IF = {f21/(f21 - f22)}L1 - {f1f2/(f21 - f22)}L2

not all integer biases being determinable, further resulting in partial, fixed integer estimate of biases.Intended accuracy of vectors is 0.1 ppm plus 0.5 cm (FCGS A-order).  All base lines were measured at least twice, once on each of two days.

OPUS Projects uses the program GPSCOM v1203.15, a simple Helmert Blocking normal equation processor which combines multiple GPS data sets that have initially been processed by the program PAGES, to form and partially reduce normal equations eliminating numerous nuisance parameters which are not generally of interest in a large global adjustment. The normal equation elements for the global parameters, those to be passed on to a combined adjustment, are written by PAGES into a normal equation file which becomes the basic input data for the program GPSCOM. One or more of these files as well as its own output normal equation files can then be processed by GPSCOM to provide a combined adjustment of the global parameters.  This program was written in FORTRAN. Predominantly ANSI standard FORTRAN, a few isolated routines query the operating system for time and date, user and system identification. These "system" routines are inherently non-portable but, should be easily modified or disabled.  For more information, go to http://www.ngs.noaa.gov/GRD/GPS/DOC/gpscom/gpscom.html

Four IGS tracking stations, including KOKB, one of the co-located techniques tied into the site survey, were normally constrained in the adjustment.  The coordinates for the tracking stations used in the adjustment were extracted from the ITRF website, http://itrf.ensg.ign.fr/, in ITRF2008 (epoch date of survey).  All points are correlated as seen in Attachment – GPS  Geocentric Coordinates and Covariances.  The result was ITRF2008 coordinates for the current survey epoch date for the six ground network points.

## Results

Table GPS-2 on the next page summarizes redundant GPS vectors from the seven onsite baselines and shows how well the repeated measurements compare.  The last column shows the total variance each vector experienced when compared to the final baseline length estimate.  In the final adjustment, each vector was weighted based on OPUS-Projects’ estimate of quality.

| Baseline |  | Final Baseline Est. (meters) | Sess. | Var. Δ X (meters) | Var. Δ Y (meters) | Var. Δ Z (meters) | Comb. Var. (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FLR2 | FLF1 | 6.397 | 38 | 0.002 | -0.005 | 0.006 | 0.008 |
|  |  |  | 39 | 0.000 | -0.014 | 0.006 | 0.015 |
|  |  |  | 40a | -0.003 | -0.002 | -0.008 | 0.009 |
|  |  |  | 40b | -0.003 | 0.029 | -0.016 | 0.033 |
|  |  |  | 42 | 0.008 | -0.002 | 0.005 | 0.009 |
|  |  |  | 43 | -0.001 | -0.014 | 0.006 | 0.016 |
|  |  |  | 45 | -0.001 | 0.002 | -0.005 | 0.005 |
|  |  |  | 46 | 0.000 | 0.003 | -0.003 | 0.004 |
|  |  |  | 47a | -0.005 | -0.008 | 0.004 | 0.010 |
|  |  |  | 47b | 0.007 | -0.014 | 0.007 | 0.017 |
|  |  |  | 48 | 0.009 | 0.014 | 0.000 | 0.017 |
|  |  |  | 49 | -0.001 | 0.002 | 0.002 | 0.002 |
| FLR1 | FLF1 | 19.635 | 38 | 0.003 | -0.002 | 0.007 | 0.008 |
|  |  |  | 39 | 0.001 | -0.011 | 0.006 | 0.013 |
|  |  |  | 40a | -0.002 | -0.004 | -0.004 | 0.006 |
|  |  |  | 40b | -0.003 | 0.031 | -0.014 | 0.034 |
|  |  |  | 48 | 0.008 | -0.005 | 0.006 | 0.011 |
|  |  |  | 49 | 0.001 | -0.015 | 0.009 | 0.018 |
| FLR3 | FLF1 | 21.157 | 38 | 0.003 | -0.004 | 0.008 | 0.010 |
|  |  |  | 39 | 0.001 | -0.012 | 0.007 | 0.013 |
|  |  |  | 40a | -0.003 | -0.002 | -0.005 | 0.006 |
|  |  |  | 40b | -0.003 | 0.030 | -0.015 | 0.034 |
|  |  |  | 48 | 0.009 | -0.003 | 0.007 | 0.012 |
|  |  |  | 49 | 0.001 | -0.017 | 0.009 | 0.019 |
| TABV | FLF1 | 246.100 | 42 | -0.001 | 0.002 | -0.004 | 0.005 |
|  |  |  | 43 | 0.000 | 0.001 | -0.001 | 0.002 |
|  |  |  | 45 | -0.007 | -0.005 | 0.003 | 0.010 |
|  |  |  | 46 | 0.004 | -0.012 | 0.007 | 0.014 |
| RCM4 | FLF1 | 139.305 | 42 | -0.001 | 0.002 | -0.006 | 0.006 |
|  |  |  | 43 | -0.001 | 0.003 | -0.004 | 0.005 |
|  |  |  | 45 | -0.006 | -0.009 | 0.003 | 0.011 |
|  |  |  | 46 | 0.005 | -0.013 | 0.005 | 0.014 |
|  |  |  | 47a | 0.007 | 0.013 | -0.001 | 0.015 |
|  |  |  | 47b | -0.001 | 0.000 | 0.001 | 0.002 |
| RCM2 | FLF1 | 297.004 | 42 | -0.002 | 0.000 | -0.005 | 0.005 |
|  |  |  | 43 | 0.000 | 0.001 | -0.002 | 0.002 |
|  |  |  | 45 | -0.008 | -0.006 | 0.003 | 0.011 |
|  |  |  | 46 | 0.003 | -0.011 | 0.006 | 0.013 |
|  |  |  | 47a | 0.009 | 0.013 | -0.001 | 0.016 |
| TF2A | FLF1 | 15.007 | 38 | 0.004 | -0.002 | 0.007 | 0.008 |
|  |  |  | 39 | 0.001 | -0.009 | 0.005 | 0.011 |
|  |  |  | 40a | -0.002 | 0.001 | -0.007 | 0.007 |

### Table GPS-2:  Comparison of redundant vectors.

The final GPS coordinates were computed independently using NGS’s OPUS-Projects web-based software.  See the following attachment for OPUS Projects output SINEX file.

## Attachment:  GPS Geocentric Coordinates and Covariances (SINEX)

%=SNX 1.00 NOA 15:243:80696 NOA 15:038:00000 15:050:86370 P    39 2 X - -

*-------------------------------------------------------------------------------

+FILE/REFERENCE

DESCRIPTION    OPUS Projects : NOAA/NOS/NGS

OUTPUT         network-mch FRAME: IGS08

CONTACT        charles.geoghegan@noaa.gov

SOFTWARE       pages.e & gpscom.e

HARDWARE       fermi (SunOS i86pc)

INPUT          Richmond IERS Site Survey GNSS data

-FILE/REFERENCE

*-------------------------------------------------------------------------------

+FILE/COMMENT

This combined adjustment was made from internal NGS files

of partially reduced normal equations (information matricies).

The data sets included in this solution are shown below.

../15038/Results/2015-038-Ag.nrm                               0

../15039/Results/2015-039-Ag.nrm                               0

../15040/Results/2015-040-Ag.nrm                               0

../15040/Results/2015-040-Bg.nrm                               0

../15042/Results/2015-042-Ag.nrm                               0

../15043/Results/2015-043-Ag.nrm                               0

../15045/Results/2015-045-Ag.nrm                               0

../15046/Results/2015-046-Ag.nrm                               0

../15047/Results/2015-047-Ag.nrm                               0

../15047/Results/2015-047-Bg.nrm                               0

../15048/Results/2015-048-Ag.nrm                               0

../15049/Results/2015-049-Ag.nrm                               0

-FILE/COMMENT

*-------------------------------------------------------------------------------

+SOLUTION/STATISTICS

NUMBER OF OBSERVATIONS                       1744265.

NUMBER OF UNKNOWNS                               3930

NUMBER OF CONSTRAINT EQUATIONS                     12

NUMBER OF DEGREES OF FREEDOM                 1740347.

VARIANCE FACTOR                     1.116749098502226

-SOLUTION/STATISTICS

*-------------------------------------------------------------------------------

+SITE/ID

*CODE PT DOMES____ T STATION DESCRIPTION___ APPROX_LON_ APPROX_LAT_ _APP_H_

flr1  - --------- P flr1                   279 36 50.8  25 36 55.4   -23.5

flr2  - --------- P flr2                   279 36 50.0  25 36 55.1   -23.6

flr3  - --------- P flr3                   279 36 50.1  25 36 55.9   -23.5

tf2a  - --------- P tf2a                   279 36 50.1  25 36 55.7   -22.1

BRMU  A 42501S004 P Bermuda, UK            295 18 13.4  32 22 13.4   -11.6

FLF1  A --------- P FL Foundation 1        279 36 50.1  25 36 55.3   -22.1

INEG  A 40507M001 P Aguascalientes, Mexico 257 42 56.9  21 51 22.2  1888.0

NLIB  A 40465M001 P North Liberty, USA     268 25 30.4  41 46 17.7   207.0

SCUB  A 40701M001 P Santiago de Cuba, Cuba 284 14 15.7  20  0 43.4    20.9

USNO  A 40451S003 P Washington, USA        282 56  1.6  38 55  8.3    48.9

tabv  - --------- P tabv                   279 36 57.4  25 36 50.8   -23.0

rcm4  - --------- P rcm4                   279 36 54.9  25 36 54.1   -23.2

rcm2  - --------- P rcm2                   279 36 57.2  25 36 48.1   -22.8

-SITE/ID

*-------------------------------------------------------------------------------

+SITE/RECEIVER

*SITE PT SOLN T DATA_START__ DATA_END____ DESCRIPTION_________ S/N__ FIRMWARE___

flr1  -    1 P 15:038:00000 15:049:86399 TRIMBLE NETR5        01631 NP 4.85 / S

flr2  -    1 P 15:038:00000 15:049:86399 TRIMBLE NETR5        01590 NP 4.85 / S

flr3  -    1 P 15:038:00000 15:049:86399 TRIMBLE NETR5        01583 NP 4.85 / S

tf2a  -    1 P 15:038:00000 15:040:86399 TRIMBLE NETR5        01584 NP 4.85 / S

BRMU  A    2 P 15:038:00000 15:049:86399 LEICA GRX1200GGPRO   35124 8.71/IME3.8

FLF1  A    1 P 15:038:00000 15:049:86399 JAVAD TRE_G3TH DELTA 698   3.5.7

INEG  A    4 P 15:038:00000 15:049:86399 TRIMBLE 5700         02202 1.24

NLIB  A    1 P 15:038:00000 15:049:86399 ASHTECH UZ-12        IR220 CQ00

SCUB  A    2 P 15:038:00000 15:049:86399 JAVAD TRE_G3TH DELTA 204   3.5.7

USNO  A    1 P 15:038:00000 15:049:86399 ASHTECH Z-XII3T      RT919 1L01-1D04

tabv  -    1 P 15:042:00000 15:046:86399 TRIMBLE NETR5        01631 NP 4.85 / S

rcm4  -    1 P 15:042:00000 15:047:86399 TRIMBLE NETR5        01583 NP 4.85 / S

rcm2  -    1 P 15:042:00000 15:047:86399 TRIMBLE NETR5        01584 NP 4.85 / S

-SITE/RECEIVER

*-------------------------------------------------------------------------------

+SITE/ANTENNA

*SITE PT SOLN T DATA_START__ DATA_END____ DESCRIPTION_________ S/N__

flr1  -    1 P 15:038:00000 15:049:86399 TPSCR.G3        NONE -1613

flr2  -    1 P 15:038:00000 15:049:86399 TPSCR.G3        NONE -1626

flr3  -    1 P 15:038:00000 15:049:86399 TPSCR.G3        NONE -1614

tf2a  -    1 P 15:038:00000 15:040:86399 TPSCR.G3        NONE -1628

BRMU  A    2 P 15:038:00000 15:049:86399 JAVRINGANT_DM   NONE 00480

FLF1  A    1 P 15:038:00000 15:049:86399 JAVRINGANT_DM   NONE 445

INEG  A    4 P 15:038:00000 15:049:86399 TRM29659.00     NONE 02201

NLIB  A    1 P 15:038:00000 15:049:86399 TPSCR.G3        SCIS 383-0

SCUB  A    2 P 15:038:00000 15:049:86399 JAV_RINGANT_G3T NONE 341

USNO  A    1 P 15:038:00000 15:049:86399 AOAD/M_T        NONE 309

tabv  -    1 P 15:042:00000 15:046:86399 TPSCR.G3        NONE -1613

rcm4  -    1 P 15:042:00000 15:047:86399 TPSCR.G3        NONE -1614

rcm2  -    1 P 15:042:00000 15:047:86399 TPSCR.G3        NONE -1628

-SITE/ANTENNA

*-------------------------------------------------------------------------------

+SITE/GPS_PHASE_CENTER

* Antenna information is from the file

*     /home/OPUS/files/ngs08.atx

* abbreviated below as  ant.info

*                            UP___  NORTH EAST_  UP___  NORTH  EAST_

*DESCRIPTION_________ S/N__ L1->ARP(m)__________ __L2->ARP(m)________ AZ_EL

TPSCR.G3        NONE -1613 0.0884 -.0002 0.0003 0.1194 0.0003 -.0000   ant.info

TPSCR.G3        NONE -1626 0.0884 -.0002 0.0003 0.1194 0.0003 -.0000   ant.info

TPSCR.G3        NONE -1614 0.0884 -.0002 0.0003 0.1194 0.0003 -.0000   ant.info

TPSCR.G3        NONE -1628 0.0884 -.0002 0.0003 0.1194 0.0003 -.0000   ant.info

JAVRINGANT_DM   NONE 00480 0.0893 0.0011 0.0009 0.1196 0.0003 -.0001   ant.info

JAVRINGANT_DM   NONE 445   0.0893 0.0011 0.0009 0.1196 0.0003 -.0001   ant.info

TRM29659.00     NONE 02201 0.0910 0.0002 0.0002 0.1204 -.0000 -.0000   ant.info

TPSCR.G3        SCIS 383-0 0.0842 -.0002 0.0003 0.1203 0.0005 -.0001   ant.info

JAV_RINGANT_G3T NONE 341   0.0490 0.0003 0.0022 0.0547 0.0068 -.0032   ant.info

AOAD/M_T        NONE 309   0.0918 0.0006 -.0004 0.1203 -.0001 -.0006   ant.info

TPSCR.G3        NONE -1613 0.0884 -.0002 0.0003 0.1194 0.0003 -.0000   ant.info

TPSCR.G3        NONE -1614 0.0884 -.0002 0.0003 0.1194 0.0003 -.0000   ant.info

TPSCR.G3        NONE -1628 0.0884 -.0002 0.0003 0.1194 0.0003 -.0000   ant.info

-SITE/GPS_PHASE_CENTER

*-------------------------------------------------------------------------------

+SITE/ECCENTRICITY

*SITE PT SOLN T DATA_START__ DATA_END____ AXE ARP->BENCHMARK(m)_________

flr1  -    1 P 15:038:00000 15:049:86399 UNE   1.5143   0.0000  -0.0000

flr2  -    1 P 15:038:00000 15:049:86399 UNE   1.5872   0.0000  -0.0000

flr3  -    1 P 15:038:00000 15:049:86399 UNE   1.5886   0.0000  -0.0000

tf2a  -    1 P 15:038:00000 15:040:86399 UNE   0.0000   0.0000   0.0000

BRMU  A    2 P 15:038:00000 15:049:86399 UNE   0.0000   0.0000   0.0000

FLF1  A    1 P 15:038:00000 15:049:86399 UNE   0.0083   0.0000  -0.0000

INEG  A    4 P 15:038:00000 15:049:86399 UNE   0.0705   0.0000  -0.0000

NLIB  A    1 P 15:038:00000 15:049:86399 UNE   0.6127   0.0000  -0.0000

SCUB  A    2 P 15:038:00000 15:049:86399 UNE   0.0460   0.0000  -0.0000

USNO  A    1 P 15:038:00000 15:049:86399 UNE   0.0000   0.0000   0.0000

tabv  -    1 P 15:042:00000 15:046:86399 UNE   1.6544   0.0000  -0.0000

rcm4  -    1 P 15:042:00000 15:047:86399 UNE   1.5043   0.0000  -0.0000

rcm2  -    1 P 15:042:00000 15:047:86399 UNE   1.6154   0.0000  -0.0000

-SITE/ECCENTRICITY

*-------------------------------------------------------------------------------

+SOLUTION/EPOCHS

*SITE PT SOLN T _DATA_START_ __DATA_END__ _MEAN_EPOCH_

flr1  -    1 P 15:038:00000 15:049:86399 15:043:36937

flr2  -    1 P 15:038:00000 15:049:86399 15:044:68146

flr3  -    1 P 15:038:00000 15:049:86399 15:043:28771

tf2a  -    1 P 15:038:00000 15:040:86399 15:039:16939

BRMU  A    2 P 15:038:00000 15:049:86399 15:044:69845

FLF1  A    1 P 15:038:00000 15:049:86399 15:044:66457

INEG  A    4 P 15:038:00000 15:049:86399 15:044:68252

NLIB  A    1 P 15:038:00000 15:049:86399 15:044:73047

SCUB  A    2 P 15:038:00000 15:049:86399 15:044:69343

USNO  A    1 P 15:038:00000 15:049:86399 15:044:72014

tabv  -    1 P 15:042:00000 15:046:86399 15:044:49394

rcm4  -    1 P 15:042:00000 15:047:86399 15:045:82499

rcm2  -    1 P 15:042:00000 15:047:86399 15:044:75225

-SOLUTION/EPOCHS

*-------------------------------------------------------------------------------

+SOLUTION/ESTIMATE

*INDEX TYPE   CODE PT SOLN _REF_EPOCH__ UNIT S  __ESTIMATED VALUE___ _STD_DEV___

1 STAX   flr1  -    1 15:044:43185 m    2 0.961125360276229E+06 .903334E-03

2 STAY   flr1  -    1 15:044:43185 m    2 -.567402403359427E+07 .238275E-02

3 STAZ   flr1  -    1 15:044:43185 m    2 0.274069299779443E+07 .147821E-02

4 STAX   flr2  -    1 15:044:43185 m    2 0.961103884295735E+06 .867242E-03

5 STAY   flr2  -    1 15:044:43185 m    2 -.567403191369579E+07 .222539E-02

6 STAZ   flr2  -    1 15:044:43185 m    2 0.274068393503235E+07 .141474E-02

7 STAX   flr3  -    1 15:044:43185 m    2 0.961105108125548E+06 .902830E-03

8 STAY   flr3  -    1 15:044:43185 m    2 -.567402035586620E+07 .239408E-02

9 STAZ   flr3  -    1 15:044:43185 m    2 0.274070765240304E+07 .148363E-02

10 STAX   tf2a  -    1 15:044:43185 m    2 0.961107962931107E+06 .999537E-03

11 STAY   tf2a  -    1 15:044:43185 m    2 -.567402379519339E+07 .294658E-02

12 STAZ   tf2a  -    1 15:044:43185 m    2 0.274070261087765E+07 .171828E-02

13 STAX   BRMU  A    2 15:044:43185 m    0 0.230470332438808E+07 .832715E-03

14 STAY   BRMU  A    2 15:044:43185 m    0 -.487481715029333E+07 .210502E-02

15 STAZ   BRMU  A    2 15:044:43185 m    0 0.339518699279702E+07 .129383E-02

16 STAX   FLF1  A    1 15:044:43185 m    2 0.961107214640020E+06 .841511E-03

17 STAY   FLF1  A    1 15:044:43185 m    2 -.567403050506231E+07 .208936E-02

18 STAZ   FLF1  A    1 15:044:43185 m    2 0.274068921088486E+07 .136511E-02

19 STAX   INEG  A    4 15:044:43185 m    2 -.126043572405143E+07 .906776E-03

20 STAY   INEG  A    4 15:044:43185 m    2 -.578854696444340E+07 .204438E-02

21 STAZ   INEG  A    4 15:044:43185 m    2 0.236033991991119E+07 .138398E-02

22 STAX   NLIB  A    1 15:044:43185 m    0 -.130934769020866E+06 .858498E-03

23 STAY   NLIB  A    1 15:044:43185 m    0 -.476229169833603E+07 .213075E-02

24 STAZ   NLIB  A    1 15:044:43185 m    0 0.422685460774885E+07 .123279E-02

25 STAX   SCUB  A    2 15:044:43185 m    0 0.147453801455090E+07 .834946E-03

26 STAY   SCUB  A    2 15:044:43185 m    0 -.581124325615002E+07 .205513E-02

27 STAZ   SCUB  A    2 15:044:43185 m    0 0.216895885001133E+07 .139673E-02

28 STAX   USNO  A    1 15:044:43185 m    0 0.111218960379731E+07 .832512E-03

29 STAY   USNO  A    1 15:044:43185 m    0 -.484295500954010E+07 .213401E-02

30 STAZ   USNO  A    1 15:044:43185 m    0 0.398535226029236E+07 .126031E-02

31 STAX   tabv  -    1 15:044:43185 m    2 0.961317554611346E+06 .912068E-03

32 STAY   tabv  -    1 15:044:43185 m    2 -.567405482401611E+07 .248512E-02

33 STAZ   tabv  -    1 15:044:43185 m    2 0.274056378475413E+07 .152569E-02

34 STAX   rcm4  -    1 15:044:43185 m    2 0.961242619751981E+06 .884981E-03

35 STAY   rcm4  -    1 15:044:43185 m    2 -.567402163302077E+07 .234361E-02

36 STAZ   rcm4  -    1 15:044:43185 m    2 0.274065769219541E+07 .146583E-02

37 STAX   rcm2  -    1 15:044:43185 m    2 0.961318712187052E+06 .912492E-03

38 STAY   rcm2  -    1 15:044:43185 m    2 -.567409096034155E+07 .247800E-02

39 STAZ   rcm2  -    1 15:044:43185 m    2 0.274048964457302E+07 .152539E-02

-SOLUTION/ESTIMATE

*-------------------------------------------------------------------------------

+SOLUTION/APRIORI

*INDEX TYPE   CODE PT SOLN _REF_EPOCH__ UNIT S  __A-PRIORI VALUE____ _STD_DEV___

13 STAX   BRMU  A    2 15:044:43185 m    0 0.230470347501943E+07 .189725E-03

14 STAY   BRMU  A    2 15:044:43185 m    0 -.487481716923134E+07 .189725E-03

15 STAZ   BRMU  A    2 15:044:43185 m    0 0.339518695107125E+07 .189725E-03

22 STAX   NLIB  A    1 15:044:43185 m    0 -.130934602972532E+06 .189725E-03

23 STAY   NLIB  A    1 15:044:43185 m    0 -.476229172154649E+07 .189725E-03

24 STAZ   NLIB  A    1 15:044:43185 m    0 0.422685465133471E+07 .189725E-03

25 STAX   SCUB  A    2 15:044:43185 m    0 0.147453808890483E+07 .195453E-03

26 STAY   SCUB  A    2 15:044:43185 m    0 -.581124326489303E+07 .195453E-03

27 STAZ   SCUB  A    2 15:044:43185 m    0 0.216895881777513E+07 .195453E-03

28 STAX   USNO  A    1 15:044:43185 m    0 0.111218977295776E+07 .189725E-03

29 STAY   USNO  A    1 15:044:43185 m    0 -.484295502578374E+07 .189725E-03

30 STAZ   USNO  A    1 15:044:43185 m    0 0.398535226585870E+07 .189725E-03

-SOLUTION/APRIORI

*-------------------------------------------------------------------------------

+SOLUTION/MATRIX_ESTIMATE U COVA

1     1 0.816012924125876E-06 -.577218578318633E-06 0.273945901565966E-06

1     4 0.714959107490512E-06 -.329405915239599E-06 0.154373952300169E-06

1     7 0.725739417056832E-06 -.353495483321123E-06 0.165948117912110E-06

1    10 0.723861865236159E-06 -.348779481743524E-06 0.164202340546206E-06

1    13 0.680344247712374E-06 -.344454261597177E-06 0.138963228946166E-06

1    16 0.705960039592318E-06 -.310512978876913E-06 0.145089399719547E-06

1    19 0.731830630001173E-06 -.297826259991467E-06 0.144254091951550E-06

1    22 0.714304419605737E-06 -.285598436135034E-06 0.904230787064059E-07

1    25 0.696325712854611E-06 -.306181070528475E-06 0.157012953149144E-06

1    28 0.696439788300211E-06 -.323341768216807E-06 0.115195660620597E-06

1    31 0.000000000000000E+00 0.000000000000000E+00 0.000000000000000E+00

1    34 0.000000000000000E+00 0.000000000000000E+00 0.000000000000000E+00

1    37 0.000000000000000E+00 0.000000000000000E+00 0.000000000000000E+00

2     2 0.567747756874192E-05 -.271865153384823E-05 -.327608734048668E-06

2     5 0.442591409325109E-05 -.211892919967134E-05 -.353368084560866E-06

2     8 0.455568098875241E-05 -.217954603006204E-05 -.352005403384274E-06

2    11 0.454611493087107E-05 -.217477606859747E-05 0.173714584774090E-08

2    14 0.433801647079710E-05 -.185863140273348E-05 -.308473653677009E-06

2    17 0.431716064568857E-05 -.206675231265404E-05 -.772476410344645E-06

2    20 0.418751471557008E-05 -.210079011772031E-05 -.517964849806376E-06

2    23 0.438213738555036E-05 -.166825475407775E-05 -.188004076214096E-06

2    26 0.422731503451960E-05 -.215599893792396E-05 -.264348162073447E-06

2    29 0.439326968125839E-05 -.174958236159827E-05 0.000000000000000E+00

2    32 0.000000000000000E+00 0.000000000000000E+00 0.000000000000000E+00

2    35 0.000000000000000E+00 0.000000000000000E+00 0.000000000000000E+00

2    38 0.000000000000000E+00 0.000000000000000E+00

3     3 0.218510499338891E-05 0.153880193778353E-06 -.211497832552241E-05

3     6 0.186360534957409E-05 0.165963405853298E-06 -.217815772788332E-05

3     9 0.189609483350615E-05 0.166414669285062E-06 -.217295183839977E-05

3    12 0.189277081173855E-05 -.356888167688976E-07 -.211975083984713E-05

3    15 0.173403481441639E-05 0.146886725853644E-06 -.207537304294091E-05

3    18 0.184314313196472E-05 0.426620667763624E-06 -.200781375327220E-05

3    21 0.186333515325013E-05 0.281637326448020E-06 -.214565201303383E-05

3    24 0.162778636333888E-05 0.708475265799929E-07 -.200512441747065E-05

3    27 0.188545566180541E-05 0.123544401356170E-06 -.214977722019819E-05

3    30 0.167315323202979E-05 0.000000000000000E+00 0.000000000000000E+00

3    33 0.000000000000000E+00 0.000000000000000E+00 0.000000000000000E+00

3    36 0.000000000000000E+00 0.000000000000000E+00 0.000000000000000E+00

3    39 0.000000000000000E+00

4     4 0.752109102165913E-06 -.426741777630574E-06 0.202514819287310E-06

4     7 0.714878829431879E-06 -.327955147563573E-06 0.154042315964779E-06

4    10 0.713779636321481E-06 -.323969487789813E-06 0.152554018707305E-06

4    13 0.680138410939803E-06 -.342662285364221E-06 0.138341901949958E-06

4    16 0.705632681613419E-06 -.308695721829708E-06 0.144388000665864E-06

4    19 0.731389610582627E-06 -.295919299353109E-06 0.143471245863848E-06

4    22 0.713939282332449E-06 -.283813754786358E-06 0.898546408206824E-07

4    25 0.696045926904669E-06 -.304384357220943E-06 0.156279697587835E-06

4    28 0.696174310633681E-06 -.321607332351305E-06 0.114717473725470E-06

4    31 0.713510080398423E-06 -.324534644898003E-06 0.152303193888830E-06

4    34 0.714029975309012E-06 -.326964142861476E-06 0.153315504646540E-06

4    37 0.713826740037790E-06 -.325828891975729E-06 0.152888268795634E-06

5     5 0.495234216462706E-05 -.237205778435380E-05 -.329546214479581E-06

5     8 0.442941162828720E-05 -.211695873031063E-05 -.329018128609024E-06

5    11 0.442055254034513E-05 -.211280105775165E-05 0.646957285761536E-09

5    14 0.432754927629633E-05 -.185104527681441E-05 -.308656477088965E-06

5    17 0.430665126153875E-05 -.205863661198469E-05 -.771321160761579E-06

5    20 0.417788780363154E-05 -.209278240662510E-05 -.517685168604615E-06

5    23 0.437150813645439E-05 -.166102033443340E-05 -.188662177652554E-06

5    26 0.421801222992581E-05 -.214801357023757E-05 -.264780663458211E-06

5    29 0.438291403565283E-05 -.174251018955964E-05 -.323852891869973E-06

5    32 0.440034517645125E-05 -.210473664855385E-05 -.326076263705912E-06

5    35 0.440662413139549E-05 -.210847787577518E-05 -.325235398447206E-06

5    38 0.440903004328200E-05 -.210942989632059E-05

6     6 0.200148603162534E-05 0.154515513313237E-06 -.211950458315450E-05

6     9 0.186401273605937E-05 0.155320646350202E-06 -.211504592744833E-05

6    12 0.186112413133604E-05 -.354632866890523E-07 -.211565328746506E-05

6    15 0.173135608334694E-05 0.146806676828221E-06 -.207130140332398E-05

6    18 0.184030034738102E-05 0.426079115277515E-06 -.200421545999720E-05

6    21 0.186057956473158E-05 0.281394418627628E-06 -.214160533927020E-05

6    24 0.162532126559681E-05 0.709100582058055E-07 -.200146037360385E-05

6    27 0.188268082927327E-05 0.123563663302291E-06 -.214580572496167E-05

6    30 0.167074418436898E-05 0.152409244221876E-06 -.210327956788720E-05

6    33 0.185763349456344E-05 0.153352456806122E-06 -.210622590989030E-05

6    36 0.185965265634367E-05 0.153116715120035E-06 -.210749437334530E-05

6    39 0.186009497653908E-05

7     7 0.815102259197697E-06 -.581081762140749E-06 0.276300095734908E-06

7    10 0.723724360149833E-06 -.349329109015266E-06 0.164591322402243E-06

7    13 0.680315197340717E-06 -.344383073793379E-06 0.138986269323672E-06

7    16 0.705928410016021E-06 -.310431697891788E-06 0.145101333307333E-06

7    19 0.731788668849473E-06 -.297775567397554E-06 0.144281047038262E-06

7    22 0.714276957438911E-06 -.285539299511083E-06 0.904535723605632E-07

7    25 0.696290243917735E-06 -.306091838970587E-06 0.157024310407250E-06

7    28 0.696410906883906E-06 -.323267550907853E-06 0.115219453119914E-06

7    31 0.707393397284419E-06 -.309992132148306E-06 0.144937002354856E-06

7    34 0.707249605599201E-06 -.310912553716884E-06 0.145176970175335E-06

7    37 0.707377758279735E-06 -.310047803349811E-06 0.144823193314463E-06

8     8 0.573160021584165E-05 -.274753108568349E-05 -.352761665128048E-06

8    11 0.455440022195327E-05 -.217816129460891E-05 0.148383263193247E-08

8    14 0.433794076840232E-05 -.185729347045130E-05 -.308631893041196E-06

8    17 0.431699212816064E-05 -.206531950162065E-05 -.772434032119121E-06

8    20 0.418790971014611E-05 -.209961276693916E-05 -.518112525403857E-06

8    23 0.438222587784332E-05 -.166702580836664E-05 -.188218570734257E-06

8    26 0.422727767828868E-05 -.215463885844001E-05 -.264534812182436E-06

8    29 0.439317254565681E-05 -.174823759478949E-05 -.307488730839130E-06

8    32 0.431784363819387E-05 -.206647705548799E-05 -.308145967541357E-06

8    35 0.431612287025032E-05 -.206593108313914E-05 -.307678620886882E-06

8    38 0.431932969829589E-05 -.206729202613821E-05

9     9 0.220116173916324E-05 0.166813882050500E-06 -.217757279866155E-05

9    12 0.189478496929328E-05 -.356118096833004E-07 -.211975336680770E-05

9    15 0.173332635165708E-05 0.146925076180276E-06 -.207533023065439E-05

9    18 0.184239106661045E-05 0.426559053272872E-06 -.200808456275967E-05

9    21 0.186272743294928E-05 0.281671700604118E-06 -.214577087227598E-05

9    24 0.162715862823997E-05 0.709169867761144E-07 -.200516224670733E-05

9    27 0.188474731962059E-05 0.123594023984498E-06 -.214979485726746E-05

9    30 0.167246002519633E-05 0.144433731115715E-06 -.206233800244714E-05

9    33 0.183532821091334E-05 0.144565683391862E-06 -.206102000095707E-05

9    36 0.183488366182768E-05 0.144458179468287E-06 -.206272904390119E-05

9    39 0.183557528126781E-05

10    10 0.999073341582585E-06 -.107552155950495E-05 0.510882454338170E-06

10    13 0.679755685778064E-06 -.344139602291839E-06 0.139947583583721E-06

10    16 0.705301312141731E-06 -.309985495491242E-06 0.145936639302651E-06

10    19 0.731393065918716E-06 -.297157432465737E-06 0.145020248483249E-06

10    22 0.713812551470797E-06 -.285579622608839E-06 0.915482000023430E-07

10    25 0.695664442799396E-06 -.305642784230565E-06 0.157905606306449E-06

10    28 0.695913143203185E-06 -.323051283811734E-06 0.116163256635596E-06

10    31 0.706739280345096E-06 -.309660680190150E-06 0.145830304565619E-06

10    34 0.706585475456388E-06 -.310573790391784E-06 0.146067125044981E-06

10    37 0.706720621916518E-06 -.309713658386334E-06 0.145714391087729E-06

11    11 0.868235252202136E-05 -.418477018963567E-05 0.398977190347884E-08

11    14 0.432981175691628E-05 -.185445065319667E-05 -.304886452329680E-06

11    17 0.430790179858238E-05 -.206133363737441E-05 -.768095807171563E-06

11    20 0.417862290607461E-05 -.209538017900291E-05 -.514258509682999E-06

11    23 0.437450141218055E-05 -.166456939729050E-05 -.184815837080559E-06

11    26 0.421771769481969E-05 -.215040232952794E-05 -.261361518896862E-06

11    29 0.438523383050277E-05 -.174574013759727E-05 -.303881574093730E-06

11    32 0.430919098175214E-05 -.206260113443211E-05 -.304531597265876E-06

11    35 0.430744150928426E-05 -.206203167258945E-05 -.304065249007279E-06

11    38 0.431065131972074E-05 -.206339602053407E-05

12    12 0.295247952502389E-05 -.364913430404458E-07 -.211626697368070E-05

12    15 0.173125009041857E-05 0.145487395196832E-06 -.207145536043678E-05

12    18 0.183983502458785E-05 0.424816843515297E-06 -.200417951234467E-05

12    21 0.186009324545253E-05 0.280168793189802E-06 -.214248053744431E-05

12    24 0.162529056467989E-05 0.696540937186667E-07 -.200102483257790E-05

12    27 0.188203901988677E-05 0.122410709791017E-06 -.214640592051988E-05

12    30 0.167057453622631E-05 0.143076866480282E-06 -.205858969498587E-05

12    33 0.183279795747749E-05 0.143206855824475E-06 -.205725557132004E-05

12    36 0.183234185793772E-05 0.143098688696085E-06 -.205896558298538E-05

12    39 0.183303492616845E-05

13    13 0.693414763977581E-06 -.586310146904880E-07 -.650255373060753E-08

13    16 0.679249309139070E-06 0.990595915475462E-09 -.358009696783192E-07

13    19 0.670032706122124E-06 0.680575937793867E-08 -.402860856500092E-07

13    22 0.671286027439079E-06 0.326893696173840E-07 -.613980592896664E-07

13    25 0.678926763295595E-06 -.185954422032247E-08 -.304966186567670E-07

13    28 0.673586671796143E-06 -.685825400943169E-08 -.403156828315207E-07

13    31 0.680257447421641E-06 0.881329452174363E-09 -.356293721327922E-07

13    34 0.679988751623812E-06 0.696171381387387E-10 -.354899517230745E-07

13    37 0.680183997307145E-06 0.101018026395811E-08 -.358514784382121E-07

14    14 0.443110752668856E-05 -.195495571721706E-05 -.343810454502684E-06

14    17 0.433171135993282E-05 -.211754390578618E-05 -.817832631972708E-06

14    20 0.420938740429359E-05 -.215669966003135E-05 -.559081711145800E-06

14    23 0.440972819736636E-05 -.171816716174499E-05 -.220451153439037E-06

14    26 0.424203697061211E-05 -.221012454870392E-05 -.301454759932843E-06

14    29 0.442672990231652E-05 -.180466342465961E-05 -.341322248584309E-06

14    32 0.432383069906486E-05 -.211465934151499E-05 -.341650529785261E-06

14    35 0.431998095558208E-05 -.211292159280352E-05 -.341216877579204E-06

14    38 0.432363601919200E-05 -.211455030113516E-05

15    15 0.167399548116145E-05 0.141111195414831E-06 -.186440852548413E-05

15    18 0.173960498015758E-05 0.400588679177176E-06 -.180464065755078E-05

15    21 0.175928093788918E-05 0.266891643594796E-06 -.193530953729440E-05

15    24 0.154445357947315E-05 0.703191496317918E-07 -.179913288657997E-05

15    27 0.177865969263235E-05 0.121062553138825E-06 -.194254418272927E-05

15    30 0.158888945308324E-05 0.138187764948237E-06 -.184870460985258E-05

15    33 0.173104874895578E-05 0.138151589227814E-06 -.184639424926486E-05

15    36 0.172997814561591E-05 0.138051524514434E-06 -.184825003845863E-05

15    39 0.173079523785334E-05

16    16 0.708141579835984E-06 -.319582443535084E-06 0.152114465613291E-06

16    19 0.731343387521757E-06 -.296952897996354E-06 0.146561446406546E-06

16    22 0.713556327887718E-06 -.285536418054693E-06 0.929458992274019E-07

16    25 0.695382699228588E-06 -.305716111838006E-06 0.159442656006077E-06

16    28 0.695597942634642E-06 -.322948504103716E-06 0.117571207490180E-06

16    31 0.705719570108328E-06 -.308019281943776E-06 0.146526984449544E-06

16    34 0.705418491361687E-06 -.308627646634790E-06 0.146609537402226E-06

16    37 0.705628318418419E-06 -.307820366040315E-06 0.146270477009840E-06

17    17 0.436541564353996E-05 -.209957576044300E-05 -.775075417064871E-06

17    20 0.418104345931579E-05 -.210736679848935E-05 -.520021984206955E-06

17    23 0.437825412412867E-05 -.167620850041113E-05 -.189546930172475E-06

17    26 0.422208730192929E-05 -.216299328345417E-05 -.266085713085476E-06

17    29 0.438837256418692E-05 -.175662531766601E-05 -.307274306242969E-06

17    32 0.430241859747715E-05 -.207003926365591E-05 -.307639806334997E-06

17    35 0.429894225459629E-05 -.206854925828417E-05 -.307206148148133E-06

17    38 0.430228905918569E-05 -.206998154996387E-05

18    18 0.186351527846882E-05 0.428113618755560E-06 -.200560262404164E-05

18    21 0.186954129986240E-05 0.282581252972006E-06 -.214479339060823E-05

18    24 0.163448746531759E-05 0.713443761306994E-07 -.200359051747683E-05

18    27 0.189197926507383E-05 0.124173080362108E-06 -.214838212185014E-05

18    30 0.167938556938615E-05 0.144165340595931E-06 -.205585466335408E-05

18    33 0.183983319745771E-05 0.144141285571785E-06 -.205358155733987E-05

18    36 0.183881704478526E-05 0.144036292848807E-06 -.205535875032375E-05

18    39 0.183955594587990E-05

19    19 0.822242033332959E-06 -.728648205895901E-06 0.423557613191886E-06

19    22 0.766006503999093E-06 -.765279407279929E-06 0.331044794783868E-06

19    25 0.707048981553108E-06 -.760280434452732E-06 0.450761581671339E-06

19    28 0.717637526910091E-06 -.803043674716749E-06 0.364398192392951E-06

19    31 0.731433495187361E-06 -.770387185754907E-06 0.425789448554362E-06

19    34 0.731145927905086E-06 -.770452035189003E-06 0.425644473112594E-06

19    37 0.731333213426584E-06 -.770103348823870E-06 0.425501291309256E-06

20    20 0.417947431156499E-05 -.209170844008767E-05 -.504201551382306E-06

20    23 0.426057722160261E-05 -.162443800866986E-05 -.177714012091190E-06

20    26 0.409315115656262E-05 -.209430813461349E-05 -.254940555439239E-06

20    29 0.426498770425046E-05 -.169846602824266E-05 -.294798143549479E-06

20    32 0.417448608749621E-05 -.200338841020691E-05 -.294792565872867E-06

20    35 0.417071466737063E-05 -.200175913661251E-05 -.294628407868389E-06

20    38 0.417411885157519E-05 -.200320536408363E-05

21    21 0.191539519217904E-05 0.285368368412606E-06 -.218665543225252E-05

21    24 0.165334760701217E-05 0.681512062338126E-07 -.203578468881877E-05

21    27 0.191374197917153E-05 0.123226460318071E-06 -.218813959559904E-05

21    30 0.169733146180746E-05 0.143356750610976E-06 -.209028597170138E-05

21    33 0.186026508971690E-05 0.143173667147298E-06 -.208776942709791E-05

21    36 0.185914913293628E-05 0.143171821688719E-06 -.208970386289108E-05

21    39 0.185994405185960E-05

22    22 0.737019243893898E-06 -.503005484422890E-06 0.207597980657878E-06

22    25 0.696823234957124E-06 -.510493252217694E-06 0.299512516926480E-06

22    28 0.702272128936180E-06 -.540930982062413E-06 0.237224343776486E-06

22    31 0.714026471985317E-06 -.517030716866508E-06 0.281162056551591E-06

22    34 0.713719671747682E-06 -.517362686269152E-06 0.281107864290923E-06

22    37 0.713907800149333E-06 -.516748360574813E-06 0.280846161178244E-06

23    23 0.454009518977237E-05 -.181109616077830E-05 -.159987596173361E-06

23    26 0.428469606019996E-05 -.223701326638706E-05 -.243463995499986E-06

23    29 0.447352607013556E-05 -.182889239619480E-05 -.282576410887476E-06

23    32 0.436800456872605E-05 -.214074991156947E-05 -.282821593500528E-06

23    35 0.436375880997650E-05 -.213890183028111E-05 -.282465742645237E-06

23    38 0.436742426482569E-05 -.214047105609763E-05

24    24 0.151977341720029E-05 0.271325654147112E-07 -.161250981664007E-05

24    27 0.166819973958640E-05 0.753454404576948E-07 -.174983213179508E-05

24    30 0.149880352029925E-05 0.897746081866995E-07 -.165893001697711E-05

24    33 0.162511011449625E-05 0.896920788147529E-07 -.165657136620459E-05

24    36 0.162407898223527E-05 0.896419462741737E-07 -.165825918331800E-05

24    39 0.162474712879650E-05

25    25 0.697135049357210E-06 -.210133129630025E-06 0.887321204516232E-07

25    28 0.686817946803175E-06 -.197393745457261E-06 0.496013357628567E-07

25    31 0.696085993068359E-06 -.188060545349274E-06 0.705610560925191E-07

25    34 0.695835039566927E-06 -.188905808990831E-06 0.707632362946837E-07

25    37 0.696010046673587E-06 -.187900688171723E-06 0.703259051634629E-07

26    26 0.422354020071847E-05 -.212430537349773E-05 -.262040548790183E-06

26    29 0.429431006719923E-05 -.169092072225467E-05 -.302827323515767E-06

26    32 0.421427983739453E-05 -.200028368448777E-05 -.303299658140705E-06

26    35 0.421122170545986E-05 -.199899075215425E-05 -.302792070594834E-06

26    38 0.421416719763077E-05 -.200024549774556E-05

27    27 0.195086748035143E-05 0.135125300215678E-06 -.224058566414678E-05

27    30 0.171459884919539E-05 0.156009674175984E-06 -.214534657831175E-05

27    33 0.188226696904584E-05 0.156008124798642E-06 -.214307629257992E-05

27    36 0.188125631414292E-05 0.155884721158179E-06 -.214479431891484E-05

27    39 0.188196351266806E-05

28    28 0.693075826192118E-06 -.296173575397226E-06 0.112142290735412E-06

28    31 0.696286024610827E-06 -.264353039077377E-06 0.123379671552513E-06

28    34 0.695999120235852E-06 -.264956533650251E-06 0.123453600698404E-06

28    37 0.696207938525071E-06 -.264130738110901E-06 0.123114248597735E-06

29    29 0.455399897082382E-05 -.188903768747610E-05 -.320336785687292E-06

29    32 0.437935810348716E-05 -.214485172234675E-05 -.320631904543627E-06

29    35 0.437555589755079E-05 -.214326496268327E-05 -.320246292550796E-06

29    38 0.437900023666856E-05 -.214472088552925E-05

30    30 0.158838112512686E-05 0.114654294741334E-06 -.174034597510000E-05

30    33 0.167048889321277E-05 0.114611971599425E-06 -.173832660292373E-05

30    36 0.166963185879807E-05 0.114539362717682E-06 -.173986282937171E-05

30    39 0.167025447234009E-05

31    31 0.831868497229094E-06 -.645889940541986E-06 0.309926144577118E-06

31    34 0.718114292766674E-06 -.336858473037801E-06 0.158854321987355E-06

31    37 0.724918461395414E-06 -.351379188098845E-06 0.165694086780894E-06

32    32 0.617582721210017E-05 -.296966743005788E-05 -.336639304339267E-06

32    35 0.447504080847534E-05 -.214088381721646E-05 -.351643752022826E-06

32    38 0.455685006036501E-05 -.218040428158632E-05

33    33 0.232773421544472E-05 0.158787035074244E-06 -.214005746154529E-05

33    36 0.187835773485283E-05 0.165964528188805E-06 -.217997416942977E-05

33    39 0.190011089272607E-05

34    34 0.783191971817967E-06 -.516267629871982E-06 0.248443523717550E-06

34    37 0.718693017899893E-06 -.338936710498888E-06 0.159948462870885E-06

35    35 0.549250325866749E-05 -.263864327466613E-05 -.339260924386678E-06

35    38 0.449015041859111E-05 -.214824523326839E-05

36    36 0.214865997707331E-05 0.160162092439600E-06 -.214857261794061E-05

36    39 0.188284551822908E-05

37    37 0.832642142400616E-06 -.638017550314349E-06 0.306441997174133E-06

38    38 0.614046090662184E-05 -.296052471548624E-05

39    39 0.232682237101829E-05

-SOLUTION/MATRIX_ESTIMATE U COVA

*-------------------------------------------------------------------------------

+SOLUTION/MATRIX_APRIORI U COVA

13    13 0.334949286010342E-04 -.308155287171888E-05 -.223909732262904E-06

14    14 0.385560174832399E-04 0.473604926513883E-06

15    15 0.405072559187602E-04

22    22 0.300125919336957E-04 0.297670283414483E-06 -.809953633697326E-22

23    23 0.408311486828012E-04 -.129437148565290E-21

24    24 0.408393328259806E-04

25    25 0.315378008718738E-04 -.387293014566051E-05 0.127283831127105E-05

26    26 0.458185361427299E-04 -.501633153768261E-05

27    27 0.344764344640754E-04

28    28 0.315051695823782E-04 -.214360088508531E-05 0.329146520147033E-21

29    29 0.403470525181889E-04 -.260648768856008E-20

30    30 0.408393328259806E-04

-SOLUTION/MATRIX_APRIORI U COVA

*-------------------------------------------------------------------------------

%ENDSNX